## 3.No

## 1️⃣ Understand the Topology (What’s Connected)

### Left side (LAN 1)

- PC0, PC1 → Hub0

- PC2, PC3 → Hub1

- Hub0 & Hub1 → Switch0

- Switch0 → Router (Gig0/0/0)

👉 All these devices are in one LAN (same network)

### Right side (LAN 2)

- PC4 → Switch1

- Switch1 → Router (Gig0/0/1)

👉 This is another LAN (different network)

The router connects two networks, so routing is required.

## 2️⃣ IP Addressing Scheme (VERY IMPORTANT)

Use two different networks.

### 🔹 Network 1 (Left LAN)

Network: 192.168.1.0/24

Subnet Mask: 255.255.255.0

| Device | IP Address | Gateway |
| --- | --- | --- |
| PC0 | 192.168.1.2 | 192.168.1.1 |
| PC1 | 192.168.1.3 | 192.168.1.1 |
| PC2 | 192.168.1.4 | 192.168.1.1 |
| PC3 | 192.168.1.5 | 192.168.1.1 |
| Router G0/0/0 | 192.168.1.1 | — |

### 🔹 Network 2 (Right LAN)

Network: 192.168.2.0/24

Subnet Mask: 255.255.255.0

| Device | IP Address | Gateway |
| --- | --- | --- |
| PC4 | 192.168.2.2 | 192.168.2.1 |
| Router G0/0/1 | 192.168.2.1 | — |

## 3️⃣ Configure the Router (MOST IMPORTANT PART)

### ▶ Click Router1

Go to CLI

#### Step 1: Enter configuration mode

enable

configure terminal

#### Step 2: Configure GigabitEthernet 0/0/0 (Left LAN)

interface gig0/0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

#### Step 3: Configure GigabitEthernet 0/0/1 (Right LAN)

interface gig0/0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

#### Step 4: Save configuration

end

write memory

✅ Router is now configured.

## 4️⃣ Configure PCs (One by One)

### ▶ For Each PC:

- Click the PC

- Go to Desktop → IP Configuration

- Select Static

- Enter values

### Example: PC0

IP Address: 192.168.1.2

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Do the same for:

- PC1 → 192.168.1.3

- PC2 → 192.168.1.4

- PC3 → 192.168.1.5

- PC4 → 192.168.2.2 (Gateway 192.168.2.1)

## 5️⃣ Switch & Hub Configuration

### ✅ Switches (2960)

- No configuration required

- Switches work by default (Layer 2)

### ✅ Hubs

- No configuration

- Just broadcast signals

DHCP setup   
dhcp-server>en

dhcp-server>enable

dhcp-server#

dhcp-server#conf

dhcp-server#configure t

dhcp-server#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

dhcp-server(config)#

dhcp-server(config)#int

dhcp-server(config)#interface g0/0

dhcp-server(config-if)#ip add 192.168.1.1 255.255.255.0

dhcp-server(config-if)#no sh

dhcp-server(config-if)#no shutdown

dhcp-server(config-if)#

dhcp-server(config-if)#

dhcp-server(config-if)#int

dhcp-server(config-if)#exit

dhcp-server(config)#int

dhcp-server(config)#interface g0/1

dhcp-server(config-if)#

dhcp-server(config-if)#ip add 192.168.2.1 255.255.255.0

dhcp-server(config-if)#

dhcp-server(config-if)#

dhcp-server(config-if)#no sh

dhcp-server(config-if)#no shutdown

dhcp-server(config-if)#

dhcp-server(config-if)#exit

dhcp-server(config)#

dhcp-server(config)#

dhcp-server(config)#ip dhcp ex

dhcp-server(config)#ip dhcp excluded-address 192.168.1.1

dhcp-server(config)#ip dhcp ex

dhcp-server(config)#ip dhcp excluded-address 192.168.2.1

dhcp-server(config)#

dhcp-server(config)#

dhcp-server(config)#ip dhcp pool 192.168.1.1

dhcp-server(dhcp-config)#

dhcp-server(dhcp-config)#

dhcp-server(dhcp-config)#net

dhcp-server(dhcp-config)#network 192.168.1.0 255.255.255.0

dhcp-server(dhcp-config)#

dhcp-server(dhcp-config)#

dhcp-server(dhcp-config)#de

dhcp-server(dhcp-config)#default-router 192.168.1.1

dhcp-server(dhcp-config)#

dhcp-server(dhcp-config)#dns-ser

dhcp-server(dhcp-config)#dns-server 0.0.0.0

dhcp-server(dhcp-config)#exit

dhcp-server(config)#ip dhcp pool 192.168.2.1

dhcp-server(dhcp-config)#net

dhcp-server(dhcp-config)#network 192.168.2.0 255.255.255.0

dhcp-server(dhcp-config)#de

dhcp-server(dhcp-config)#default-router 192.168.2.1

dhcp-server(dhcp-config)#dns-ser

dhcp-server(dhcp-config)#dns-server 0.0.0.0

dhcp-server(dhcp-config)#

OSPF Routing

enable

configure terminal

router ospf 1

network 192.168.10.0 255.255.255.0 area 0

network 192.168.11.0 255.255.255.0 area 0

network 192.168.12.0 255.255.255.0 area 0

network 10.0.0.0 255.0.0.0 area 0

network 20.0.0.0 255.0.0.0 area 0

exit

new

!R1 Configuration

en

config t

hostname R1

int se2/0

ip add 192.168.12.1  255.255.255.0

no sh

int s3/0

ip add 192.168.14.1 255.255.255.0

no sh

int fa0/0

ip add 192.168.1.1 255.255.255.0

no sh

router ospf 1

int se2/0

ip ospf 1 area 0

int se3/0

ip ospf 1 area 0

int fa0/0

ip ospf 1 area 0

————

!R2 Configuration

en

config t

hostname R2

int se2/0

ip add 192.168.12.2  255.255.255.0

no sh

int s3/0

ip add 192.168.23.1 255.255.255.0

no sh

router ospf 1

int se2/0

ip ospf 1 area 0

int se3/0

ip ospf 1 area 0

——

!R3 Configuration

en

config t

hostname R3

int se2/0

ip add 192.168.23.2  255.255.255.0

no sh

int s3/0

ip add 192.168.34.1 255.255.255.0

no sh

int fa0/0

ip add 192.168.2.1 255.255.255.0

no sh

router ospf 1

int se2/0

ip ospf 1 area 0

int se3/0

ip ospf 1 area 0

int fa0/0

ip ospf 1 area 0

————

!R4 Configuration

en

config t

hostname R4

int se2/0

ip add 192.168.34.2  255.255.255.0

no sh

int s3/0

ip add 192.168.14.1 255.255.255.0

no sh

router ospf 1

int se2/0

ip ospf 1 area 0

int se3/0

ip ospf 1 area 0